# DESIGN FOR PLACES OF WORK

# WORKPLACE (HSW) REGULATIONS 1992 PROCEDURE

| Date | July 2026 |
| --- | --- |
| Author | Simon Archer |

Contents

1. Purpose	3

2. Scope	3

3. Legal context	3

4. When this procedure applies	3

5. What we check	3

6. How we do it	4

7. Roles and responsibilities	4

8. Records and evidence	4

9. Review	4

## 1. Purpose

This procedure explains how ⟦legalName⟧ (⟦short⟧) makes sure that, when our design work creates or affects a place where people will work, the design takes proper account of the Workplace (Health, Safety and Welfare) Regulations 1992. It applies to all our work as Designer.

## 2. Scope

It covers all ⟦short⟧ design and specification work, and in particular the design of security and CCTV control rooms, monitoring and reception positions, equipment and communications rooms, and the way our equipment and cabling sit within occupied buildings.

## 3. Legal context

The Workplace (Health, Safety and Welfare) Regulations 1992 set out duties for the working environment, covering matters such as lighting, ventilation, temperature, room space, workstations, floors and traffic routes, and welfare facilities. As a designer we also have duties under the Construction (Design and Management) Regulations 2015 to eliminate or reduce foreseeable risks, and we take account of the Building Regulations and of the Health and Safety (Display Screen Equipment) Regulations 1992 for operator positions.

## 4. When this procedure applies

We apply it at the earliest design stage, and again at each design review, on any project where our work affects a place of work. Typical examples are a new or altered control room, a manned monitoring or reception position, or the placement of equipment in an occupied space.

## 5. What we check

For each relevant design we consider the following and record our findings on the Design for Places of Work Checklist:

•  Lighting: control and monitoring positions have suitable and sufficient lighting, camera and equipment positions do not create glare or shadow, and our work does not reduce general or emergency lighting (Regulation 8).

•  Ventilation: equipment heat loads are allowed for and our equipment or cabling does not obstruct vents, grilles or air paths (Regulation 6).

•  Temperature: heat from racks, recorders and power supplies in control and communications rooms is considered and managed (Regulation 7).

•  Room dimensions and space: there is enough space at operator positions and around equipment for safe use and maintenance (Regulation 10).

•  Workstations and display screens: monitoring positions allow a safe and comfortable operator set-up, taking account of screen position, seating and viewing distance (Regulation 11 and the Display Screen Equipment Regulations 1992).

•  Floors and traffic routes: cable routing and containment do not create trip hazards or obstruct routes and escape ways (Regulation 12).

•  Doors, gates and access control: security measures do not impede safe movement or emergency escape (Regulations 17 and 18).

•  Welfare and other facilities: our work does not compromise existing welfare facilities or required space (Regulations 20 to 25).

•  Coordination: any residual issues are shared with the Principal Designer, Principal Contractor and Client.

## 6. How we do it

•  Design brief: the scope and the working environment are agreed with the client at the outset.

•  Design review: the checklist is completed and dated at design review, before drawings or specifications are issued.

•  Coordination: relevant findings are communicated to other duty holders using our Design Risk Communication Form.

•  Records: completed checklists, design notes and correspondence are kept on the project file in line with our Information Management (Golden Thread) Policy.

## 7. Roles and responsibilities

•  Managing Director: overall responsibility for this procedure.

•  Project Lead: ensures the checklist is completed and that any issues are actioned and communicated.

•  Design staff: apply the checks and flag concerns promptly.

## 8. Records and evidence

The completed and dated Design for Places of Work Checklist, together with any related design notes and correspondence, provides the evidence that these checks have taken place.

## 9. Review

This procedure is reviewed once a year, and sooner if the law or our way of working changes.

⟦hsLead⟧

Managing Director, ⟦legalName⟧ 				Date: 15/07/2026

| Version | Date | Author | Description of Changes | Approval By |
| --- | --- | --- | --- | --- |
| 1.0 | July 2026 | Simon Archer | First issue | ⟦hsLead⟧ |