⟦legalNameUpper⟧

Toolbox Talk Fire Integrity

Contents

Purpose	3

Key Points	3

On-site Examples	3

Why?	3

Action Points	3

Questions	3

# Purpose

How quality of the ductwork installation directly affects fire safety, air tightness, and the overall compliance of the building.

Key Points
The Building Regulations 2010 (Part B and Part F) and the Building Safety Act 2022 require that all ductwork systems perform as designed for fire resistance and air quality.
Even small shortcuts – like poor sealing, missing fire collars, or incorrect damper orientation – can cause a system to fail inspection or, worse, fail in a fire situation.
The amended building regulations require competent installation and documented verification of all fire-rated components.

On-site Examples
Ensure all fire dampers are installed are accessible for maintenance, and sealed with approved fire-rated materials as specified.
Check penetrations – any hole through a wall or slab must be sealed with tested fire-stopping products, never expanding foam unless specifically approved.
Maintain air tightness – ensure duct joints are sealed to DW144 standards as specified.

Why? 
Failing to meet fire integrity standards can delay handover, lead to failed inspections, or legal penalties.
Poor quality ductwork can spread smoke or contaminants, putting lives at risk when the building is occupied.

Action Points
Check all penetrations, seals, and dampers against the drawings.
Never substitute materials without written approval.
Complete all fire-stopping QA records.
Treat fire-rated ducts and walls as life safety systems, not just building fabric!

## Questions

| Name | Date | Signature |
| --- | --- | --- |