| Talk reference |  |
| --- | --- |
| Date delivered |  |
| Site / project |  |
| Delivered by |  |
| Number of attendees |  |

Spill prevention and response

Why this matters to us

We don’t carry a lot of fuel or chemicals on our jobs, but we DO use mastic, sealants.  A spill that ends up in a drain, in soil or in a watercourse can lead to a prosecution, a clean-up bill in the thousands, and the principal contractor losing trust in us. The Environment Agency has the power to prosecute the company AND the individual.

What can spill on our jobs

- Mastic and sealant — usually small but messy and hard to clean up.

- Cutting and machine lubricants from workshop or hand tools.

- Diesel or petrol from a vehicle, especially after an accident.

- Cleaning products and solvents.

Prevention — the easy bit

- Store containers upright, lidded, and away from drains. If you can move it indoors or under cover, do.

- Use drip trays under anything that might leak (hand-held machines, jerry cans).

- Don’t leave open containers on top of skips, scaffolds or near gully covers.

- Check the spill kit at the start of each job — know where it is and what’s in it.

- Park company vehicles where any leak can be seen and contained, not over a drain.

What to do if it spills

Three steps, in this order:

- 1. STOP THE SOURCE. Tip the container upright, close the valve, plug the leak. Whatever it takes to make it stop spreading.

- 2. CONTAIN IT. Get the spill kit. Pads soak up oil and fuel; granules absorb everything; the drain mat seals a gully cover. Surround the spill before it reaches a drain or soil.

- 3. REPORT IT. Tell the supervisor straight away, even if it’s small. The supervisor tells the principal contractor. The supervisor tells the office.

NEVER hose a spill into a drain or down a gully. NEVER wash it into the soil. The drain leads either to a watercourse or to a treatment works — both are pollution offences.

What to do if it’s reached a drain or watercourse

Stop work in the area. Tell the supervisor IMMEDIATELY. The supervisor will tell the principal contractor and decide whether the Environment Agency needs to be called (24-hour incident hotline 0800 80 70 60). The faster we report, the better the outcome.

Discuss as a team

Take a few minutes:  Where is the spill kit on this job?  When did we last check it?  Are there any drains or watercourses near where we’re working?  Anything missing?

Attendance and confirmation of understanding

By signing below, each attendee confirms they attended this toolbox talk, had the chance to ask questions, and understand what is expected of them on ⟦legalName⟧ projects.

| Name (printed) | Signature | Date |
| --- | --- | --- |

Talk delivered by (signature): _________________________________   Date: ___ / ___ / ______

Actions or follow-up

1. _________________________________________________________________________________

2. _________________________________________________________________________________

3. _________________________________________________________________________________